TECHNICAL DOCUMENTATION

 Test Setup

Simulation: Gazebo lawn world

Flight stack: PX4 Autopilot

Communication: UDP sockets

Linux IP: 172.20.10.2

Port: 6000

Network: Mobile hotspot

LAUNCH SETUP PROCESS

Step 1 — Launch Simulation

- Gazebo lawn world was started

- x500 drone model spawned successfully

- PX4 SITL initialized and connected

Step 2 — Start Communication

- UDP receiver executed on Linux terminal

- UDP sender executed on Windows terminal

Step 3 — Send Command

- Command TAKEOFF sent from Windows system

Step 4 — System Response

- Linux terminal received and displayed message

- Acknowledgment sent back to Windows

- PX4 simulation remained active with drone visible

Gazebo Simulation

- Lawn environment loaded

- Drone visible in simulation

- PX4 successfully connected

FOR CONNECTION BETWEEN LINUX AND WINDOWS:

We have use UDP to transfer data.

Linux Receiver (Ubuntu):

import socket

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

sock.bind(("0.0.0.0", 6000))

print("Linux ready...")

while True:

data, addr = sock.recvfrom(1024)

msg = data.decode()

print("Received from Windows:", msg)

reply = "ACK: " + msg

sock.sendto(reply.encode(), addr)

Windows Sender (Command Node):
We will push commands from windows, such that the drones will fly/land/hover accordingly.


import socket

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

sock.settimeout(5)

ip = "172.20.10.2"

port = 6000

msg = input("Enter message: ")

sock.sendto(msg.encode(), (ip, port))

try:

data, addr = sock.recvfrom(1024)

print("Reply from Linux:", data.decode())

except:

print("No reply received")


QNX TECHNICAL DETAILS: